| Name. _______________________________ Student Admn. No.: _______________________ |  |  |  | Printed Pages:01 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| School of _______________ Back Paper Examination (Odd and Even Semester) – July - August 2024 [Programme: BTech CSE ] [Semester: IV ) [Batch: ] |  |  |  |  |  |  |  |  |
| Course Title: DATA COMMUNICATION & NETWORKING Course Code: BCSE2012 |  |  |  | Max Marks: 100 Time: 3 Hrs. |  |  |  |  |
| Instructions: |  | 1. All questions are compulsory. - Assume missing data suitably, if any. |  |  |  |  |  |  |
|  |  |  | K Level |  | COs |  |  | Marks |
| SECTION-A (15 Marks) 5 Marks each |  |  |  |  |  |  |  |  |
| 1. | Discuss the roles and responsibilities of MAC sub-layer OR LLC sub-layer. |  | K1 |  |  |  | 5 |  |
| 2. | a. Define bit rate and baud rate. The bit rate of a signal is 300 bps. If each signal element carries 6 bits, what is baud rate? b. A binary signal is sent over a 3 KHz channel, whose SNR is 20 dB. What is the maximum achievable data rate?............ SNR = ? |  | K2 |  |  |  | 5 |  |
| 3. | Assume we have downloaded text documentation at the rate of 100 pages per second. A page is an average of 24 lines with 80 characters per line. If we assume that one character requires 8 bits, compute the bit rate |  | K2 |  |  |  | 5 |  |
| SECTION-B (40 Marks) 10 Marks each |  |  |  |  |  |  |  |  |
| 4. | Interpret the significance of the Presentation Layer in the OSI model and outline its primary responsibilities. |  | K2 |  |  |  | 10 |  |
| 5. | What is the purpose of installing router in network, why router build routing tables. Discuss two routing algorithms in detail. |  | K3 |  |  |  | 10 |  |
| 6. | Explain the role of the Session Layer in the OSI model and discuss its key functionalities. |  | K3 |  |  |  | 10 |  |
| 7. | Construct and compare the TCP/IP and OSI-reference model in detail. |  | K3 |  |  |  | 10 |  |
| SECTION-C (45 Marks) 15 Marks each |  |  |  |  |  |  |  |  |
| 8. | a. Discuss different available IP addressing schemes. b. An organization is granted the block 16.0.0.0/8. The administrator wants to create 500 fixed-length subnets. i. Find the subnet mask. ii. Find the number of addresses in each subnet. iii. Find the first and the last address in the first subnet. c. Find the subnet mask in i. 1024 subnets in class A ii. 256 subnets in class B |  | K4 |  |  |  | 15 |  |
| 9. | Define the term "Congestion" in the network. Explain the different reasons of occurrence of congestion, also discuss the two approaches of control. |  | K5 |  |  |  | 15 |  |
| 10 | Model the process of establishing a TCP connection, including the role of SYN and ACK packets, sequence numbers, and the three-way handshake. Discuss the significance of each step in ensuring reliable communication between the client and server. Provide a detailed explanation, including the actions taken by both the client and server during the connection establishment phase. |  | K5 |  |  |  | 15 |  |